Test Plan

ECE485/585 Final Project Group 7

Deven Bishnu, Bhavana Manikyanahalli Srinivasegowda,

Lokarjun Ramesh, Xiang Li

Functions to Test:

Syntax:

Input: Input files must be opened successfully and correctly interpreted. Tests should include lines for each valid event type (e.g. read request, clear cache, print cache contents) and a wide range of addresses (including the maximum and minimum valid addresses).

Output: Output syntax must exactly match the format in the Final Project assignment document. This can be tested by running a diff on the output against the expected output format for busops, snoops results, and higher-level cache communications. Event 9 (print cache contents) should print cache contents and state in a concise but readable format (qualitative measurement).

Mode identification: Operating mode should be correctly identified by testing the program in all operating modes, and recording the result of the function identifying it.

Usage Statistics:

Cache reads: Read requests should result in correctly identifying whether the address requested is present in the cache, and should increment the cache miss or cache hit counter by 1 (as appropriate) as well as incrementing the cache read counter. Requesting data not in the cache that conflicts with data in the cache should result in the old data being evicted per an LRU replacement policy. Tests should include addresses absent from the cache, addresses present in the cache, addresses recently evicted from the cache, and maximum and minimum valid addresses.

Cache writes: Write requests should result in correctly identifying the location in the cache to write to, measured by whether or not that location in the cache changes after the request. A request should evict conflicting data and follow an LRU replacement policy. A request should increment the cache write counter by 1, as well as incrementing the cache miss or cache hit counter by 1 (as appropriate). Tests should include addresses absent from the cache, addresses present in the cache, addresses recently evicted from the cache, and maximum and minimum valid addresses.

Cache hits: Total must be correct after some amount of instructions. Tests should include an event set with only cache hits, an event set with only cache misses, and an event set with a mix.

Cache misses: Total must be correct after some amount of instructions. Tests should include an event set with only cache hits, an event set with only cache misses, and an event set with a mix.

Cache hit ratio: Value must be correct based on values of cache hits and cache misses.

Snoops/BusOps/L2 Communications:

BusOps: Printed contents of BusOp must be correct (by value, not by format). Tests should include all types of busops and a wide range of addresses, including the maximum and minimum possible addresses.

Snoops: Printed contents of snoop result must be correct (by value, not by format). Tests should include all types of snoop result (HIT/NOHIT/HITM) and a wide range of addresses, including the maximum and minimum possible addresses. This should be replicated for snoop read, snoop write, snoop read with intent to modify, and snoop invalidate events.

L2 Communications: Printed messages to higher-level caches must be correct (by value, not by format). Tests should include all types of communication (int 1-4) and a wide range of addresses, including the maximum and minimum possible addresses.

Cache contents: Cache contents should be correct in terms of occupied/dirty blocks, as well as the contents of each occupied block: specifically the byte offset, index, tag, valid, MESI state, and LRU replacement bits. A variety of configurations should be tested using random addresses, including at least 1 each for the maximum and minimum addresses, as well as testing at least 1 eviction. Requires read, write, and evict functions to be tested beforehand.

0. Outline

This test plan will include the following:

-Scope: What areas will be tested

-Objectives: What each area will be tested for

-Deliverables: What the results of each test should look like

-Strategy: How each area is going to be tested

1. Scope

The following areas will be tested:

-Syntax: Correct input identification, correct output format

-Usage statistics

-Snoops/BusOps/L2 communications

-Cache contents: Cache contents must be correct after all prior instructions

2. Objectives

This test plan must assess the following for each area:

Syntax:

-Input syntax: correctly identifying commands and addresses

-Output syntax: text of print statements must exactly match requirements, print statements of contents and state of cache must be concise and readable

-Modes: Simulation must correctly identify and operate in correct mode based on runtime arguments

Usage Statistics:

-Cache reads: Count must be correct for each cache

-Cache writes: Count must be correct for each cache

-Cache hits: Count must be correct for every cache

-Cache misses: Count must be correct for every cache

-Cache hit ratio: Value must be correct based on hits and misses

Snoops/BusOps/L2 communications

-Snooped read request:

-Snooped write request:

-Snooped read with intent to modify request:

-Snooped invalidate command:

-L2 communications:

Cache contents:

Print statements of contents and state of cache must reflect what the cache should look like after all prior instructions

-Organization: byte offset, index, tag, valid, MESI state, and LRU replacement bits must be correct for a given instruction

3. Deliverables

Syntax:

-Input syntax: Method reading trace file should identify argument (0-9) and address (000000-ffffff)

4. Strategy

Syntax:

Input syntax will be conducted through unit tests on the method reading trace files. These will assess the input (whether the method is interpreting the text correctly) and response (whether the outcome of the method matches expectation - for instance, calling a method to handle the issued command).

Output syntax will be conducted through unit tests on all methods printing results. Inputs to these methods will be predetermined, since only the format of the output is important for this test.

Mode testing will be conducted through a unit test on the method identifying operating mode. The expected output of the method should be reflected in the unit test, whether that entails calling a different method or setting a variable.