Table of Data Structures ECE 585 Final project

| Data Structure | Description |
| --- | --- |
| CacheLine_t | Represents a cache line with tag, valid bit, MESI state |
| - valid | Valid bit (1 bit) |
| - tag | Tag bits (12 bits) |
| - mesi_state | MESI state (2 bits: I, S, E, M) |
| CacheSet_t | Represents a cache set containing multiple cache lines |
| - lines[16] | Array of 16 CacheLine_t structures |
| - lru_state | Pseudo-LRU bits for replacement (15 bits) |
| Cache_t | Represents the entire cache consisting of multiple sets |
| - sets[16384] | Array of 16,384 CacheSet_t structures |
| CacheStats_t | Holds cache statistics counters |
| - read_count | Number of cache reads |
| - write_count | Number of cache writes |
| - hit_count | Number of cache hits |
| - miss_count | Number of cache misses |
| Bus Operations | Functions and data for simulating bus operations |
| - BusOperation() | Simulate bus operations |
| - GetSnoopResult() | Get snoop results from other caches |
| - PutSnoopResult() | Report our snoop results |
| MessageToCache() | Communicate with higher-level cache |