Note: No change to any states if NOHIT

/* Bus Operation types */

#define READ  1 /* Bus Read */

#define WRITE  2 /* Bus Write */

#define INVALIDATE  3 /* Bus Invalidate */

#define RWIM  4 /* Bus Read With Intent to Modify */

/* Snoop Result types */

#define NOHIT  0 /* No hit */

#define HIT  1 /* Hit */

#define HITM  2 /* Hit to modified line */

/* L2 to L1 message types */

#define GETLINE 1   /* Request data for modified line in L1 */

#define SENDLINE  2 /* Send requested cache line to L1 */

#define INVALIDATELINE 3 /* Invalidate a line in L1 */

#define EVICTLINE  4

/* Evict a line from L1 */

// this is when L2's replacement policy causes eviction of a line that

// may be present in L1.   It could be done by a combination of GETLINE

// (if the line is potentially modified in L1) and INVALIDATELINE.

| Current MESI State | Action | Bus Operation | Snoop Result | Next MESI State | Notes |
| --- | --- | --- | --- | --- | --- |
| M (Modified) | Local eviction | FlushWB(Write) | - | I (Invalid) |  |
|  | PrRd | - | - | M (Modified) |  |
|  | PrWr | - | - | M (Modified) |  |
|  | Snooped Upgr/Inv | - | - | - | Not possible |
|  | Snooped Read | FlushWB(Write) | HITM | S (Shared) |  |
|  | Snooped ReadX | FlushWB(Write) | HITM | I (Invalid) |  |
| E (Exclusive) | Local eviction | - | - | I (Invalid) |  |
|  | PrRd | - | - | E (Exclusive) |  |
|  | PrWr | - | - | M (Modified) |  |
|  | Snooped Upgr/Inv | - | - | - | Not possible |
|  | Snooped Read | Flush(Write) | HIT | S (Shared) |  |
|  | Snooped ReadX | Flush(Write) | HIT | I (Invalid) |  |
| S (Shared) | Local eviction | - | - | I (Invalid) |  |
|  | PrRd | - | - | S (Shared) |  |
|  | PrWr | Upgr/Inv | - | M (Modified) |  |
|  | Snooped Upgr/Inv | - | - | I (Invalid) |  |
|  | Snooped Read | - | HIT | S (Shared) |  |
|  | Snooped ReadX | - | - | I (Invalid) |  |
| I (Invalid) | Eviction | - | - | - | Not possible |
|  | PrRd | Read | NoHIT | E (Exclusive) |  |
|  |  | Read | HIT/HITM | S (Shared) |  |
|  | PrWr | ReadX | - | M (Modified) |  |
|  | Snooped Upgr/Inv | - | - | - |  |
|  | Snooped Read | - | NoHIT | I (Invalid) |  |
|  | Snooped ReadX | - | - | - |  |

# Case analysis

## 0 read request from L1 data cache

CALL FUNC process_read_request_data_cach(input logic [ADDRESS_WIDTH-1:0] address)

Decode the address

readcount ++

Check for lines that are valid & same tag

IF Hit, return data to L1,

MESI doesn’t change,

Update LRU,

hit counter ++

ELSE(A MISS)

Do a BusRd, Snoop Bus

IF HIT, Get data form bus

Find space (can be packed as function)

IF full , call LRU to find a line to evict (message L1 evict),

[ In eviction, if M, Bus write, than message L1 evict ]

Else,set valid bit, load in the empty space,

Change state to S, message L1 sendline

IF HITM Get data from bus,

Find space

Change state to S, fill line, message L1 sendline

IF NOHIT Read form DRAM

Find space

Change to E, fill line , message L1 sendline

# 1 write request from L1 data cache

CALL FUNC process_read_request_instruction_cache(input logic [ADDRESS_WIDTH-1:0] address)

Decode the Address

Write count++

Check for lines that are valid & same tag

IF write HIT （M,S,E)

IF in S, change to M state

Do bus INVITE

Set dirty bit

Update LRU

IF in M, remain

Set dirty bit

Update LRU

IF in E, turn to M

Set dirty bit

Update LRU

Else write MISS

Bus RWIM

Change state to M , set dirty bit, sendline L1

2 read request from L1 instruction cache

Same

3 snooped read request

Check valid and tag if the address exists.

Put snoop result (HIT,HITM,NOHIT)

IF HIT , the line in E state -> S, bus write, Read form DRAM

S -> S ,Bus write from DRAM

IF HITM M ->S, Bus write, clear dirty bit

NoHit.

4 snooped write request (Saw an FlushWB,Flush)

Search for the line

“Should” have no effect

5 snooped read with intent to modify request

Search for the line

Put snoop result (HIT,HITM,NOHIT)

If HIT , E -> I, Invalid command to L1

S->I,  Invalid command to L1 (send text)

If HITM,M ->I , Bus Write(FlushWB), [Getline+inv] or [Envict] ?

NoHit

6 snooped invalidate command

Search for the line

Put snoop result(HIT,HITM,NOHIT)

If HIT E -> I, Invalid command to L1

S->I,  Invalid command to L1 (send text)

If HITM,M ->I ,  Invalid command to L1

NoHit

8 clear the cache and reset all state

9 print contents and state of each valid cache line